Nozzle cleaning machine operating  instructions

Basic Specifications
1. Ultrasonic Frequency: 40KHz
2. Supply Voltage: AC220V ± 10% 50Hz
3. Input power: P <100W
4. Cleaning nozzle types: 500,126,128 ...

One Pavilion, Hall II, III and IV tube pipe nozzle tube, according to the fifth nozzle tube to draw liquid when used in conjunction with or as part of the cycle.

1. Function keys

(1)Power indicator is not in use

(2) Run 1       I. II sprinkler pipe work

(3) Run 2     One, two, three, four sprinkler pipe work

(4) Ultrasonic  Ultrasonic switch

2. Work Status Description

1.Consider some of the cleaner nozzle due to only one-way flow of liquid, so the only way cleaning cycle. Cleaning fluid from the nozzle pipe through four nozzle flow into steel tanks, then the fifth pipe the liquid withdrawn from the steel tank.

2.If used only two nozzle tubes, "Run 2" button without pressing.

3. For cleaning process with ultrasonic function, press the "Ultrasonic" button to open the ultrasound; to be more than half tank of liquid when opening ultrasound; ultrasonic continuous opening time is preferably not more than half an hour.

4. When cleaning the tank if low level, it may be appropriate to mention the fifth tube to control the liquid level in the tank up.

5. Four buttons on the front panel, are open by the first time , press to close again.
3. General Operation: 
(1) to prepare a clean bottle of cleaning fluid, a waste bottle. (Note: The panel interface of two channels, one is for clean wash fluid, another is for fluid output connected to dirty water bottles.

(2) the appropriate choice of cleaning nozzle holder, nozzle and take a good external cleaning fluid and waste associated pipelines, according to a good power supply can be switched after.

(3) Depending on the number tube nozzle needed, press the "Run 1" button; if you need four sprinkler pipe, it simultaneously pressing the "Run 1" and "Run 2" button.

(4) After the dirty liquid to be basically clean, and then take away the waste container, two tubes are connected Clean cleaning fluid cleaning cycle.
(5) appropriate to start ultrasonic , Make sure the tank when attention is turned more than half the liquid.
(6) If the nozzle wash well, remove the nozzle, for nozzle maintenance treatment. Turn off the power. If the nozzle not clean, and can continue cleaning.

Free warranty provisions

- Caused by the failure of this product within six months from the date of sale, in the case of the product itself quality, rely on the factory for free repair send the relevant receipt, expired charge appropriate repairs.

- During the warranty due to the user's negligence, improper use and any unauthorized repair, adjustment, break up, reform and other damage caused by unauthorized not belong within the free warranty.

- Damage caused by the user's environment and the voltage does not meet the requirements resulting product description does not belong within the warranty period free warranty.

- Want users to take good care of the warranty-related receipts, not make up lost, unauthorized alteration void.

- Warranty transport costs borne by the user.